Processing whole blood in Smart Tubes with Lyospheres

- Allow Smart Tube pouch to equilibrate to room temperature prior to processing.

- Open Smart Tube pouch.  Each pouch contains 3 Smart Tubes, each of which is labeled at the top of the tube, just under the cap, according to the substance contained in the lyosphere:

  - 001: Excipient only

  - 020: Interferon Alpha (IFNα)

  - 300: Lipopolysaccharide (LPS)

- Label Smart Tubes appropriately. Label the side of the tube with a marker and affix a pre-made label to the other side. Secure pre-made label with clear tape to ensure it does not fall off during cryogenic storage.

- Draw peripheral venous blood into a sodium-heparin blood collection tube (BD catalog # 366480).

- Store blood at room temperature until ready to process.  Process blood within 1 hour of draw (within 30 minutes of draw for best results).

- Open each Smart Tube and remove the small plastic insert with the loop using forceps, making sure the lyosphere stays inside the tube.

- Rotate/gently shake the tube so the lyosphere falls into the tube through the hole created by removing the insert.

- Invert blood collection tube 5 times to ensure proper cell suspension. Dispense 900μL blood into each Smart Tube.

- Cap each Smart Tube, and gently shake the tube while inverting several times to ensure complete mixing.

- Incubate the Smart Tubes at 37°C for 15 minutes (use a water bath for best results).

- Remove the Smart Tubes from the water bath and break the stabilizer-filled ampoule inside by squeezing and bending the tube.

- Shake the tube gently, but firmly to mix the blood with the stabilizer.

- Incubate blood with stabilizer for 10 minutes at room temperature.

- Immediately freeze the Smart Tubes at -80°C.  If you cannot store the tubes immediately, keep them on ice until you can, but no more than 15 minutes.  Do not allow over-fixation!